# Altium Guide

## Create new project and add new documents

- File  New  Project  save Local/ in Workspace

- Right click on project  Add new to project  Schematic/ Layout etc.

## Start with schematic/ Manufacturer Part Search

- Panels  Manufacturer Part Search

  - Usefull to search existing parts

  - If you know the name of the part

  - Type in this exact name or number

  - Green IC symbol means: schematic symbol and a footprint

## Part placement and properties

- Double left click to get to properties of the part

- While placing a part: click tab to get to properties

  - Then you see the parameters

  - Footprint Show

  - Switch between 3D - model and footprint

- In Partsearch, you can also type in keywords.

  - E. g for a capacitor: Capacity, form factor, max voltage etc.

  - Or you can search for packages if you use IC’ s

- Right click  Place

  - First it needs to download the information about the part

  - Place with left click

- Change comment to e. g 10u. Part number is usually in the comment, if you delete the number from the comment, it is still stored in the properties of the part.

- Flip components with x and y while pressing the left mouse button

- Space to rotate

- For unconnected pins: crosses, else there would be an ERC error

## Wiring

- Click Ctrl + W

  - To change angle/ direction of wire: Left click

- Powerflags: GND, VCC etc.

## Annotation/ numeration of parts

- Tools  Annotation  Annotate Schematics

- Select the order etc.

- Accept Changes (Create ECO)

## Electrical Rule Check

- Go to project, right click, Validate PCB Project

- If there is an error: Panels  Messages

## Shortcuts

- Repour All: T, G, A

- Place polygon in polygon (Solid Region): P, R

- Show whole connection/ conductor track: Hover over it + Tab

- Toggle bottom and top layer: * (Numpad)

- Change between all layers: Ctrl + Shift + Mouse wheel

- Place Vias: P, V

- Change to properties from object while placing: Tab

  - Altium is like “paused”

  - Not only for placing components but for almost everything (e. g tracks).

- View configuration panel: L (in layout 3D - window)

  - Under “View options”: “Show 3D Bodies” (On/ Off to show or hide 3D models). Also possible with: Shift + Z.

  - Other settings are possible, e. g transparency of the layers and more.

- Annotate schematics: T, A, A

  - Then: Update Changes List, Accept Changes (Create ECO)

- Update PCB with components from schematics: D + U (in schematics window)

- Lock components together: Select them, right click, U (Unions)

- Polygon Pour with round corners: P, G to place Polygon, Shift + Spacebar to toggle the corner modes.

  - Space: Mirror the corner to 180°

- Select whole shape if one line is clicked: Tab (e. g for rectangles in the PCB Editor)

- Board shape from object: Mark object, D, S, D

- Align Component Text: Mark component, A, P

- Design Rules (e. g change track width): D, R

- Change Units: V, U

- Place net label: P, N

- Align two pads (so that they are at the same height): Select both pads, A, V

  - Important: After pressing A, V, a green cursor appears. Now you can decide, which part/ pad should be moved and which should stay at the same position.

- Flip the PCB in the 3D – Viewer: V, B

- Layer View: L

- Toggle Units PCB – Designer: Q

- Arrange Within Rectangle (Components): T, O, L

- Design Rule Check: T, D

- Reset Error Markers: T, M

## Settings

- Cross Select Mode: Shift + Ctrl + X

  - Marked components in the schematic or the layout are also marked in the other window.

  - Like in KiCad

- Drag Panels to Projects Panel, then it is permanently available.

- Repour polygon automatically: Preferences, PCB Editor, General, Polygon Rebuild, Repour Polygons After Modification

## PCB Layout Replication

- Mark the layout to replicate, press T, PCB Layout Replication

- Then this window appears:

## Create schematic/ layout library and new symbols/ footprints

- Create new Library: F, N, L

  - Library Type: File (Locally)

  - Select Schematic or PCB Library, Create

  - Create both Libraries and save them at the same location

- New symbol from scratch: open library, add

  - Start drawing the symbol

- New symbol from existing symbol:

  - Download the existing one (from manufacturer part search or supplier)

  - Open it

  - Select it

  - Copy, paste

  - Start drawing the symbol

  - Make sure the Grid is the same

- New footprint from scratch: open library, add

  - Start placing pads and more.

- New footprint from existing footprint

  - Download the existing one (from manufacturer part search or supplier)

  - Open it

  - Select it

  - Copy, paste

  - Start placing pads and more

  - Make sure the Grid is the same

## Link library to component panel

- Open component panel

- Go to:

- Add library

## Sheet symbols

- Generate sheet symbol: D, Y

- Select the schematic from which you want the sheet symbol

- Use ports to have connections across different sheets.

## OutputJob

- Right click on project, Add New To Project, Output Job File

## Fabrication Outputs

- Generate Fabrication Outputs: F, F, Gerber X2 Files

  - View different Layers of Layout

  - Panels, CAMtastic

## Schematic Template

- Open a project or file

- Right click, Preferences, Data management, Templates

- Add

## Special Strings

- There are special strings which adjust their shown value automatically

  - E. g ProjectName

- Own special strings: Right click, project options, parameters

## Assembly Drawing

- Go to the layout of the PCB

- File, New, Draftsman Document

- Export to PDF

## Bill of Materials

- Open BomDoc

- Reports, Bill of Materials

- Export